1-3.1 Lens Carriage

The Lens Carriage is consisted with mirror and lens, which will reflective and focuses beam on material. In general 2.0” lens is standard shipping with the machine, beside we are also offer 1.5”, 2.5”, 4.0” lenses, which selection depends on your application, the installation please refer to Mercury Users Guide.

If you find wavy line on X-axis, that probably is due to the small roller or U roller attrition.

To remove the small roller: unscrew the roller screw (LH012011A) than replace new one on the previous position, because the small roller is not symmetrical, so that please make sure the direction is correct while you fix the roller on the carriage.

To replace U roller: please notice that while the U roller is removed, the carriage only hangs by X-belt so that holds the carriage by hand to avoid extension X belt too much.

Caution: While you replace the U roller or Small roller, please check the bearings roll evenly, if not you need to examine the X-belt is fixed at the same level, beside make sure the U roller fixed parallel and equality with Small roller.

| # Lens Part List |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Part Name | Part No. | Q’ty | Remark |
| 1 | 1.5” Focus Tool | LH01357A | 1 | 8 USD/PC |
| 2 | 2.0” Focus Tool | LH01357A | 1 | 8 USD/PC |
| 3 | 2.5” Focus Tool | LH01357A | 1 | 10 USD/PC |
| 4 | 4.0” Focus Tool | LH01357A | 1 | 11 USD/PC |
| 5 | 1.5” Lens | BKM1317B | 1 | 177 USD/PC |
| 6 | 2.0” Lens | BKM1318B | 1 | 166 USD/PC |
| 7 | 2.5” Lens | BKM1319B | 1 | 172 USD/PC |
| 8 | 4.0” Lens | BKM1320B | 1 | 169 USD/PC |
| 9 | U Roller Assembly | BKM01230A | 1 | 55 USD/PC |
| 10 | Small Roller | BKM1201A | 1 | 4 USD/PC |
| 11 | Auto Focus Assembly | BKM1306B | 1 | 13 USD/PC |