# 1-8.1 Laser Tube and Basic Maintenance

Laser tube is a precise and delicate device; if it needs to be removed for service purpose, please handle it with extra care.

In general, laser tube does not require displacement for general maintenance, but if you find laser power either lost significantly or not outputting at all, and also are sure all optics device (mirrors and lens), including the output lens on laser tube, are clean and in good condition, the laser tube will need to be removed for repair.

One important maintenance tip is that the output lens on laser tube (situates in the front of tube, can be reached through the bracket hole) may be covered by dust after a long period of usage.  Dust or debris will impair laser’s output level significantly.    Therefore, it is strongly recommended to clean the laser lens on regular basis.  To clean the lens, drop optics solution on cotton bud and rub the lens gently.

On the other end of the tube, there are two LEDs, one green and one red.  The green LED indicates the power input of DC32V, where the red one shows the status of laser firing and interlocks controlled by top and front door.  If laser is not firing, and green LED is not on, there is probably a problem in the DC power source.  When in standby status with all doors closed, the red LED will half illuminate.  If any of the doors is opened or switch cable is broken, the red LED will extinguish.  And when doors are closed and a firing signal presents, the red LED will fully light up.

Before you disassemble the laser tube for service, unplug all of its cables, including the BNC cable, at the rear, which transfers firing signal from M/B, the firing protection (interlock) cable connected to a relay PCB near system mainboard inside the right side panel, and the black and red power cables from power supply.   If the firing protection cable is disconnected, laser will fire even doors are opened.  This is useful when aligning the laser beam, but should only be practiced by trained technicians.

Laser BNC Cable

When packing (make sure it is properly packed with original supplier’s packing box) and returning laser tube for service, do not include the front and rear mounting brackets, as they are not needed for service purpose.