1-9 Z Axis Working Table

1-9.1 Z Axis Working Table and Adjustment

# How to detect and adjust working table parallel to the Motion System Frame?

If material’s cut edge is not vertical or engraving effect is not even at each corner on the working table, the laser beam may not be in the center of the lens (please refer to section 2 Optical Alignment) or the working table is not parallel to the Motion System Frame.

If laser beam is sure to pass through the center of lens, please follow the instruction below to adjust the working table to the correct position.  It is recommended to have two persons to do the job together, as the back panel is required to be removed for easy work.

- Put the focus tool to carriage.

- Move the working table, by pressing up & down arrow key, until it almost touches the focus tool.

- Move the carriage to each corner of the table and check if the gaps, at all corners, between the focus tool and table are similar.  If the gaps measured are too much different (the tolerance is 0.5mm), you have to adjust working table parallel to motion frame to eliminate the difference.

- Unscrew the Z motor bracket (PS01308A), than disconnect the timing belt to separate three screw bars that support the working table.

- You can rotate screw bar to adjusting height one by one and then check the gaps by moving carriage with focus pin on.

- Once the distance of the gap is less than 0.5 mm, then you can fix the belt and motor back to normal position.  However you should be very careful while fixing the belt back onto the pulley, it is easier to have another person to hold the screw bar from the rear to prevent them from rotating.  Any rotation of the screw bar will set the table off level to the Motion System Frame.