2-2 Laser Beam and Red Beam Alignment

Front

Target

Laser Beam

Red Beam

Laser Tube

Prism Mount of Red Beam

## Fig 2-2

How to Re-alignment Laser Beam and Red Beam Parallel

- To remove the side covers and prism mount for mirror 1.

- To put a target paper away from the machine about 3 M.

- Switch on the machine and select “check laser source” in the Diagnostic Process, besides setting the power to 5% on 25W machine, for 100W or 12W please reduce or enlarge the power percentage.

- To Press Enter till the target burning the tiny point.

- To adjust Prism Mount for Red Beam mirror to joint the red beam and tiny point together.

While the process is done please re-assembly the Prism Mount for mirror 1 back. to previous position.

Laser Tube

Prism Mount of Red Beam