Section 5-1  	Electrical  Diagnostics

This section provides means to enhance system diagnostics in hardware aspect; the electronics parts in particular.

If there is an abnormality in system function, and you can not access the Hidden Diagnostic Procedure, such as a short circuit in system that will not let the machine start up, the following diagnostic chart will assist you step by step.

However, it is strongly recommended that only a qualified technical personnel is allowed to carry out the test.

Electronic Parts Diagnostic Flowchart

Power off Mercury

Remove side panel

Unplug all connectors from the mainboard.  Power on, measure power board output

DC40V+ and DC5V+ are detected at Power Board output

No

Change Power Board

Yes

Power Off and plug Control Panel (JP3) and Power (JP6) back to m/b, Power on.

No

Change mainboard (provided Control Panel, DRAM module and Flash ROMs are in good condition).

LED illuminates & LCD displays messages normally

Yes

The Working Table is going down 60 mm.

Check the Z-Motor cable connectivity, or Up/Down Limit Switch do not jammed. If the press AUTOFOCUS Table only goes down, please check probe in the focus tool.

No

Yes

Power Off, Plug X-motor cable (JP15) back to m/b, then power on

No

Yes

Check X-motor cables’ connectivity, or change X-motor if it is confirmed to be bad (see Section 5, System Related Problem).

Lens Carriage stops at the most right position, LCD displays “Initializing, Please Wait”

Power Off, Plug Y-motor cable back to JP8 & JP5 (both must be connected back !!), then power on.

Machine completes initialization step and LCD displays normally

No

Yes

Check Y-motor cables’ connectivity, or change Y-motor if it is confirmed to be bad (see Section 5 System Related Problem).

Power off.  Connect Rotary Attachment Motor (JP9) back to m/b.  Power on.

Machine completes initialization step and LCD displays normally

No

Yes

Check Rotary Attachment motor cables’ connectivity, or change the motor if there is one and is confirmed to be bad.

Power off.  Connect Laser Diode (JP7) back to m/b.  Power on.

Machine completes initialization step and LCD displays normally.  Red beam appears.

Check Diode Laser Module’s connectivity, or change Diode Laser Module if it is confirmed to be bad.

No

Yes

Power off.  Connect Laser Firing Control  PCB (JP12) back to m/b.  Power on.

Machine completes initialization step and LCD displays normally.  Red beam appears.

No

Check Door Switch’s connectivity (at both top and front door) .

Yes

Machine is in good condition in electrical aspect.  Don’t forget to connect Laser Control (J1) and Serial Port  and Parallel Port back before operating machine!