# Section 5  Troubleshooting

Laser Related Problem

## No Laser Output

- Check if all door switches are working, their cables are not broken or loose.

- Open the back chassis of laser compartment.

- Go to “test laser source” under hidden diagnostic procedure and press “Enter” to fire laser.

- Make sure the green LED on the left end of laser tube chassis (Synrad laser, view from the back of the unit) is on which indicates 32V DC power input.  If no 32V DC is measured directly out of the laser power supply, change laser power supply unit.

- Observing if the red LED is flashy as you firing the laser by pressing Enter key.

- If red LED is not flashing even the green LED is on (32V DC is present), check if the laser signal cable is loose either on laser tube or the system mainboard.

- If the cable is not the problem and there is signal output from the mainboard, change the laser tube.

## Laser Stops After a Period of Cutting Job

- Laser tube maybe overheated due to poor ventilation.  Add a cooling fan at the back of the laser or cooling the ambient temperature.

## Weak Laser Output

- Make sure the lens focus is correctly set.  Incorrect focus, such as wrong option of focal depth, could result in weak output.  Always use the correct focus pin to check or to do “Auto focus Tuning”.

- Check if all optics is properly cleaned and aligned.  Dirty or damaged lens and reflective mirrors (including laser tube’s output lens) and poor alignment will weaken the laser.

- If optics is sure not the problem, and laser drops to more or less half of the original power level, especially for 25 and 30W laser, change laser tube.  There are 2 RF boards, high frequency device, in side 25 and 30W laser, when one of the RF boards becomes defective, laser power level will be come half or less than half of the original one.  Power Wizard, a handheld power meter is handy for measuring laser power level.

System Related Problems

## System Fails at Initialization

System could not complete the initialization procedure, and the display may be blank, or system keeps trying to start up.  Change the mainboard.

## Flashing Power LED

- The X-motor may be malfunction, and short-circuiting.  Measure the impedance between pin 1 & 3 of X-motor’s driver chip (L6203), which provides PWM (Power Width Modulation, the power signal) to motor.  The ideal resistance should be from 2Ω ~ 20Ω.  If the resistance is 0Ω, there could be a short circuit in X-motor.  Change X-motor.

- If problem persists, change mainboard, the driver chip on the mainboard may be damaged.

## “X-motor Error” Message

- Power off the machine, move lens carriage left and right by hand see if it moves freely.  The error may be due to carriage’s rollers or rail surface are worn or blocked by dirt on the rail, or X-motor’s transmission belt is over tightened and impairs the motor’s mobility.

- If system keeps showing “X-motor Error” message after clearing the above-mentioned possibilities, check if its flat cable is bad connected or defective.

- If nothing wrong with the flat cable, change X-motor’s PCB (the small circuit board that connect to the flat cable).

- Change X-motor, if the PCB is fine, an open circuit in X-motor could create such problem.

- Change mainboard, if X-motor is proved to be fine.  Motor’s driver chip could be faulty. In stead of short circuit, it may have open circuit in the driver chip.  Thus, the impedance between Pin 1 & 3 of X-motor’s L6203 could be limitlessly big.

## “Y-motor Error” Message

- Power off the machine, move X-rail back and forth by hand see if it moves freely.  The error may be due to rail’s bearing or rails surfaces are worn or blocked by dirt on either sides of Y-rail, or Y-motor’s transmission belt is over tightened and impairs the motor’s mobility.

- If “Y-motor Error” message keeps playing up, after clearing the above-mentioned possibilities, check if Y-motor’s cables are bad connected or defective.

- Change Y-motor, if its cables are fine.

- Change mainboard, if Y-motor is proved to be fine.

## “Graphic Was Clipped…” Message

- The size or location of graphic image may be bigger or beyond legal working area.  Do not place graphic object, especially vectors, right from 0,0 position, or 0 at either x or y axis of working area on application software, Corel Draw for instance, even vector line’s width has been set to the thinnest.  Because at thinnest line width, it still goes beyond the boarder and causes the error.

- If the message appears randomly but frequently even image object is smaller or within the legal boarder, check or change DRAM module, a bad contact or faulty DRAM could cause such error.

## “Command Error, Use GCC driver or HP Pen Plotter Command” Message

- Make sure the driver used is for Mercury from GCC, or driver for HP pen plotter if not using GCC’s driver for control.

- If GCC driver is used and error message still appear, DRAM module may be defective.  Change DRAM module.

## Incorrect Working Area, or Homing at Wrong Position.

The problem is often seen after changing mainboard.  If fact, its not a problem at all, the new mainboard is fresh without any data stored in the EEPROM where initial data is kept, such as boundary of working area.  To correct it, you need to set an origin point to the system.  The procedure is as follow:

- Hold both Enter and Start keys at the same time while powering on the machine.

- Jog the red point (carriage) by arrow keys to the inner junction of rulers at top left corner.

- Press Enter key after the position of red pointer is right on the spot, restart the machine.

## Mechanical Related Problem

## Auto Focus Pin is Not Functioning

- The focus pin could be stocked by greasy residue that coats on it.  Clean the probe with alcohol or acetone.

- Check the cable of focus pin, there might be a bad contact or breakage.

## Table Can Not Move Up And Down

- Check if Z-motor is functioning.

- Chang mainboard, Z-motor’s driver chip could be burned.

## Weak Laser at Right Side of Working Table

- Laser beam path may be off alignment, as longer distance the laser beam travels, the more serious the laser power lost.  If this is the case, laser power drop at the lower right hand corner will be more obvious.  Realign the laser.

- If power drops evenly on right half of the table in stead of the lower right corner, the working table could be uneven against motion system’s frame.  Level the table (see the next problem item).

## Unleveled Working Table

When table is not leveled to the motion system frame, laser’s focal point will not fall uniformly at the same level across the to-be-engraved object; thus, you will always find one particular area has weaker laser.   Tune working table in parallel to motion system frame (see chapter 1-9).

#### Poor Output Quality

## Wiggling Vertical Engraved Line

- The surface of X-rail or rollers on lens carriage could be coated by dust or engraving residues.  Clean both X-rail and all rollers regularly.

- The small rollers, on the lens carriage, sit underneath X-rail, could be worn.  Change the small rollers.

## Wavy Vector Cutting Line

- Check if software resolution set to higher one can improve the problem.

- Output the image file in HPGL format directly under DOS mode without going through LaserPro Mercury driver.

- Tune Y-motor and Lens Carriage’s belt tension (all 5kg), as well as X-motor’s if problem is not improved.

#### Miscellaneous Problems

## Blurry Red Beam or Fail Red Pointer

Change Red Pointer Module.  It is necessary to realign the red beam with laser after changing the Red Pointer.

## Image breaks and shifted

- Make sure PC’s printer port mode is set to Standard or SPP or Normal mode.  EPP or ECP mode may cause data lost and shift partial image to other location.

- Check if DRAM is causing problem by changing it with a known good one.  Often a defective DRAM will cause the problem appear in the same location repeatedly.

- If graphics shift is not due to the unsuitable printer port mode, and appears randomly, the problem is possibly caused by signal interference.  If this is the case, change the signal cable, or try with another PC.  And it is advised to link the PC and LaserPro Mercury to the same ground.  Separate ground is subject to signal noise.  If LaserPro is sharing the power source with high power equipment, such as an electric drill or saw, it is strongly suggested to separate the power source.

## Image distorted or drifted toward one direction

When graphics distorted and drifted gradually toward right or left, it indicates something is wrong in X-motor.  However check if the lens carriage is loosing it tension.  If belt tension is fine, change X-motor.

## Rotary Attachment Does Not Operate Correctly

- Check if anything wrong with Rotary Attachment’s cable both the one on Rotary Attachment itself and the one from mainboard.

- Change mainboard, the driver chip for Rotary Attachment’s motor could be faulty.

## Smoke Can Not Be Vented Properly

Some material produced smoke or odor easily.  Wood, for instance, can produce lots of smoke and wood dust, which can easily clog Odor Reduction Unit’s filter sponge situating at the exhaust end.  When that happens, smoke will not be vented out and optics is likely to be covered by burned dust, on which laser can cause damage.   Therefore, it is important to keep the exhaust system ventilating effectively.

## Output Is Not Precise

Do the Scaling under Paper Option of LaserPro Mercury Driver (refer to User Guide).