2-4 Laser Beam Adjusts on X,Y Motion System and Lens Carriage

# How to adjusts Laser Beam parallel with Y-Axis

- To stick a paper in front of the Hole of the Mirror 3 (Fig2-9).

- To Move the X-Axis Rail to rear of the X-Y Motion System.(Fig 2-7)

- To press ENTER key waiting the paper burning a tiny point.

- To move the X-Axis Rail in front of the X,Y Motion System.(Fig 2-8)

- Again press ENTER key waiting the paper burning a tiny point.

- To adjusts the Mirror 2 and also refer to the Red Beam Point moving the laser Beam close to one point.

- Reset the step 1~5 till the tow point joint together. If so, which means the Laser Beam parallel with Y-Axis.

How to adjusts Laser Beam to each corner of the X,Y Motion System.

- To stick a paper in front of the Mirror 4, (On the top of the Lens Carriage)

- To move the Carriage to the P1 position.(Fig2-1)

- To press ENTER key waiting the paper burning a tiny point.

- To move the Carriage to the P2 position.(Fig2-1)

- To press ENTER key waiting the paper burning a tiny point.

- To adjusts the Mirror 3 and also refer to the Red Beam Point moving the laser Beam on P1 and P2 close to one point as near as possible.

- According to step 2~6 till the burning point close to one point from P1 to P4.

- Remove the stick and re-stick another paper below of the nozzle.

- To move the Lens Carriage to P1, than put a plastic mirror (Avoids the Laser Beam reflective by glass mirror) below the Lens Carriage on the working table.

- To press Enter key, meanwhile you can see the position of the burning point from the plastic mirror. Re-do the process from P1~P4, if the burning point P1~P4 are not only close to the one point but also in the center of the nozzle, than the Whole procedure is done. If the burning point is not in the center, please re-align Mirror 1 and check the result and adjusts according to the above method, that is important to the cutting quality, if the burning point is not close to the center, than the cutting edge will not vertical.

Fig 2-7

Front

Front

Fig 2-8

Mirror 3

Fig 2-9

Hole  Laser Beam

X-Rail